Freescale’s Path Finder

In connection with Freescale’s Cup Competition

Team XLNC

Thapar University

Patiala

# Contents

Acknowledgment

Abstract

- Introduction                                                                                                   3

- Major components                                                                                           4

- Control system 									       5

- Software design 									       9

- The source code                                        					     12

- Development tools 								               44

- Sampling time                                                                                              47

Acknowledgment

At first we would like to thank Freescale for providing us with the opportunity of implementing and improving our knowledge about microcontrollers and providing us the platform to work on your HC12 series microcontrollers

We would also like to thank Thapar University for their support at each and every phase of the Freescale Smart Car Competition.

We are also very thankful to Mr. Vishal P Arora and Mr. Sumit Miglani for supervising our work and helping us with our problems that we faced time to time.

Team XLNC

Abstract

In this report we discuss the SMART CAR, which was designed to participate in the competition path follower meeting the rules and parameters set for the competition. Besides the hardware necessary to sense, he adapted a speed control based on PID technique, implemented in a microcontroller HC12 family.

The Smart car is a car line follower consists of the electronics with a microprocessor HC12 family, which is responsible for controlling the car through the interpretation of the signals received from IR sensors,which will track a black line marked on a white track or platform, that road must be traveled in the shortest possible time, in addition to turn and climb a particular slope.

Introduction

The path  follower was made in connection with the Freescale’s  smart car race -2010. The robot automatically detects its path (in this case a black line)  and follow it by positioning itself on the track as per the programme written in it.the black line is sensed by the array of 8 sensors positioned in the front of the robot. The sensor data is then changed into the digital form by the ATD (analog to digital) cconverters. The ATD data is then processed which gives us the current position of the robot with respect to the track and then a command is given to the servo motor connected to the front wheels to guide the car on the specified track. The robot is so designed that it can change its speed according to the sharpness of the turns. It slows down more on sharp turns and runs at a fast speed when it senses a straight path.

The robot uses freescale’s mc9s12xd256 microcontroller which has HC12  architecture. The robot is a rear wheel drive driven by a single DC motor and the direction of the robot is controlled by a servo motor which controls the turning of the front wheels. Moreover the robot is provided with a pulse encoder which gives the exact value of the rounds of tires per unit time and can be used effectively to control the speed of the robot or the PWM duty of the DC motor not depending on the battery life remaining .

Major components

The  Major components of the smart car are listed below:

- Circuit board

- DC Motor

- Shaft Encoder

- Rear wheels

- Battery holder

- Chasis

- Front wheels

- IR Sensors

- Servo Motor

Control System

The control system that we used in our car is a PID controller or proportional–integral–derivative controller which is a control loop feedback mechanism. It calculates an “error” value as the difference between a measured  process variable and a desired setpoint. In this case the measured process variable is the current position of the black line and the setpoint is the desired position of the black line.

| Sensor | Sensor 6 | Sensor 5 | Sensor 4 | Sensor 3 | Sensor 2 | Sensor 1 | Sensor 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Position | 3072 | 2560 | 2048 | 1536 | 1024 | 512 | 0 |

As we are quite new to the fields of robotics so we decided to implement the default algorithm initially and will improve it eventually. The position of  any  sensor is 2*ATD* sensor index. So the position of the 7 sensors in use is given by the following table

As observed in the mobile phone camera, only three sensors are on the black line(25 mm). So the position algorithm should contain only these three ‘sensitive zones’. Although at first we decided to implement the algorithm without any change, but still logic in mind forced us to reconsider the clumsy and error prone bubble sort algorithm. So we finally come with a very simple algorithm that just finds the max value and assumes that the sensor lies in the middle of the black line

Some significant errors in position were as in case of following data

| 3 | 0 | 0 | 0 | 0 | 51 | 231 |
| --- | --- | --- | --- | --- | --- | --- |

Position according to default algorithm is

And estimated real position is ..

Let the index of sensor having highest ATD value be X, substantially decreasing the time required when sorting with bubble sort and getting the three indexes.

And the position of the black line is given by :

Position of black line = (X)*2*ATD resolution level +(compensated reading of index(X+1) – compensated reading of index (X-1))   [if 0<X<6]

Position of black line = compensated reading of index (1)       [if X=0]

Position of black line =3076- compensated reading of index (6)

ATD resolution is  256

The desired position of the black line or the setpoint is: ( position of  highest index – position of lowest index)/2 = (3072 - 0)/2 = 1536

And the error  = position of center line – setpoint

The controller attempts to minimize the error by adjusting the process control inputs. But the PID parameters used in the calculation must be tuned according to the nature of the system. The PID controller calculation  involves two separate parameters,the proportional, the integral and derivative  denoted P, I and D. The proportional value determines the reaction to the current error, the integral value determines the reaction based on the sum of recent errors, and the derivative value determines the reaction based on the rate at which the error has been changing .By tuning the three constants in the PID controller algorithm, the controller can provide control action to the servo motor and direct it for an effective path following.

The PID control scheme is named after its three correcting terms, whose sum constitutes the manipulated variable (MV). Hence:

### Proportional term

The proportional term makes a change to the output that is proportional to the current error value. The proportional response can be adjusted by multiplying the error by a constant Kp, called the proportional gain.

The proportional term is given by:

Pout  =  Kp  e(t)

Kp = proportional gain

e(t) = error at present

A high proportional gain results in a large change in the output for a given change in the error. If the proportional gain is too high, the system can become unstable . In contrast, a small gain results in a small output response to a large input error, and a less responsive  controller. If the proportional gain is too low, the control action may be too small when responding to system disturbances.

### Integral term

The contribution from the integral term is proportional to both the magnitude of the error and the duration of the error. Summing the instantaneous error over time gives the accumulated offset that should have been corrected previously. The accumulated error is then multiplied by the integral gain and added to the controller output. The magnitude of the contribution of the integral term to the overall control action is determined by the integral gain, Ki.

The integral term is given by:

where

Iout: Integral term of output

Ki: Integral gain, a tuning parameter

e: Error = Setpoint – process variable

t: Time or instantaneous time (the present)

τ: a dummy integration variable

The integral term (when added to the proportional term) accelerates the movement of the process towards setpoint and eliminates the residual steady-state error that occurs with a proportional only controller. However, since the integral term is responding to accumulated errors from the past, it can cause the present value to overshoot the setpoint value (cross over the setpoint and then create a deviation in the other direction).

### Derivative term

The rate of change of the process error is calculated by determining the slope of the error over time (i.e., its first derivative with respect to time) and multiplying this rate of change by the derivative gain Kd. The magnitude of the contribution of the derivative term (sometimes called rate) to the overall control action is termed the derivative gain, Kd.

The derivative term is given by:

where

Dout: Derivative term of output

Kd: Derivative gain, a tuning parameter

e: Error = SP − PV

t: Time or instantaneous time (the present)

The derivative term slows the rate of change of the controller output and this effect is most noticeable close to the controller setpoint. Hence, derivative control is used to reduce the magnitude of the overshoot produced by the integral component and improve the combined controller-process stability.

Software Design

The programme starts with initialization process where it initializes all its variables and macros. The a variable stop_value is created and initiated to 0 . This variable limits the robot to run only two laps and then stop. After that the programme  goes into an infinite while loop including the main body of the programme. The while loop starts with compensation of the sensor values obtained from the ATD converter. Then the sum of these compensated values is taken and decision is made depending on the total sum of the compensated values. If the total sum is greater that 600 and less than 800 then there may be a start stop line or a cross  on track. If the                                                               then there is a start stop point and  the stop_value is incremented and for stop_vaue=2 the dc motor stops and the programme ends as the robot have already ran 2 laps other wise the programme gives the control to else loop of the previous if condition. The ATD values are stored in an array out of which the sensor showing the maximum reading is slected and its index is returned .  And according to the index of the sensor with highest value speed of the robot is set and the position of the car is calculated as per the formula given above. Then the error from the setpoint is calculated which is corrected  by the PID control and the servo motor is given the desired duty cycle to guide the robot on its path. For convenient tuning of the car the Kp, Ki and Kd values were stored in EEPROM and it can be changed with the help switch 1 and switch 2. The PD controller was also implemented in the pulse encoder section and it helped to keep the speed of the car constant.

The detailed flow chart of the software is given below :

Start

All initialization and declaration

Stop_value=0

While(1)

Compensate Reading

If  600<total sum<800

and correction<25

Stop_value++

If Stop_value=2

Dc_motor_stop

Stop

Servo_set

P I D

Correction

Find error

Find position

Control Speed

Sort Array

The Source Code

- The main file

#include <hidef.h>

#include "mc9s12xd256.h"

#include "global.h"

#include "atd.h"

#include "compensation.h"

#include "dcmotor.h"

#include "uart.h"

#include "servomotor.h"

#include "eeprom.h"

#include "pid.h"

#define MAX_CORRECTION 305

#define MIN_CORRECTION -305

#define GUARD_VAL 80

#define START_CROSS           650

#define CUT_OFF_LR        250

#define START_NON90_SURE_START       1450

#define ALL_WHITE             20

#define ITERM_LIMIT 20000

#define SET_POINT 1500

#define KP_DC 3

void testmode(void);

unsigned char com_data[8];

void main(void)

{

unsigned char index;

unsigned int position=0;

int error=0,perror=0;

//Declaration related to PID control systems.

int iterm=0;

unsigned int servo_output=1500;

float correction;

int i_stop;

unsigned int sumleft,sumright,totalsum;

int start_count=0;  //variable to be removed..

// dc pid control

unsigned int pulse_count=0;   //16 bit regsiter for accumulator

int error_dc=0;     //error can be +ve or -ve

int correction_dc=0;          //+ve or -ve

unsigned char speed_dc = 80;

unsigned char count=0;

Leds_and_Switches_Init();

IEE1_Init();

//EEPROM_reset();

EEPROM_read();

ISR_init();

ATD_init();

pulse_counter_init();

dc_motor_init();                     // Current pwm duty is 85

servo_init();

//uart0_9600_init();            //To be removed in the final version of the software.

while(START)

{

//uart0_tx_string(" :::::::::::::: I'm now in main Loop !! :::::::::::: ");

//This function compensates the sen_data array and puts the result into com_data array

compensate_value(sen_data,com_data);

// Placing it here, will increase the lag between value read and action taken.. hence

// making the system unresponsive ...

ATD0CTL5_SCAN=0;          //This will start a new conversion ....

//code for PID of dc motor

if(count >=30)

{

count=0;

pulse_count =  PACN10;

PACN10=0;    //resets the pulse counter

error_dc = setpoint - pulse_count;

correction_dc =(int) ( KP_DC*error_dc);

if(speed_dc + correction_dc>=200)

{

speed_dc=200;

}

else if(speed_dc + correction_dc <=0)

{

speed_dc =0;

}

else

{

speed_dc +=correction_dc;

}

dc_motor_speed(speed_dc);

}

count ++;

//START STOP DETEECTION

//119	15	116	221	134	52	189

sumleft = com_data[0]+com_data[1]+com_data[2];

sumright = com_data[4]+com_data[5]+com_data[6];

totalsum = sumleft+sumright+com_data[3];

if(totalsum>START_CROSS)    //either CROSS or start ...

{

//SUM OF SENSORS IS LARGE OS EITHER A CROSS OR START..

if(totalsum<START_NON90_SURE_START)

{

//SO NOT ALL SENSORS ARE NOT ON BLACK .....

// SO EITHER WE HAVE A NON 90 CROSS OR START

if((sumright-sumleft) < CUT_OFF_LR && sumright-sumleft>0 || (sumleft-sumright) < CUT_OFF_LR && sumright-sumleft>0)

{

// SUM LEFT AND SUM RIGHT IS ALMOST SAME..

//SURELY WE HAVE START ..

start_count ++;

LED1=~LED1;

if(start_count>delay_dc)

{

dc_motor_stop();

}

}

else

{

// SUM OF LEFT AND RIGHT IS NOT SAME

// SO THIS IS SURELY A NON 90 CROSS

start_count=0;

LED2=~LED2;

}

}

else

{

// QUITE A LARGE SUM OF ......

//SURE 90 CROSS

//DO NOTHING

start_count=0;

LED3=~LED3

}

}

else if(totalsum<ALL_WHITE)

{

i_stop++;

if(i_stop>=100)     //STOP TIME

{

dc_motor_stop();

}

}

else

{

start_count=0;

index=sort_array(com_data);

i_stop=0;                             //To be removed in the final versions of the code

/*

if(com_data[index]<15)

{

//aa white

}

*/

//GUARD 1

if(index ==0 && com_data[index]<GUARD_VAL)

{

servo_set (1500+MIN_CORRECTION);

LED3=LED_ON;

}

//GUARD 2

else if(index==6 && com_data[index]<GUARD_VAL)

{

servo_set (1500+MAX_CORRECTION);

LED3=LED_ON;

}

else

{

//LED3=LED_OFF;

position =  (index * 512) + (com_data[index+1]-com_data[index-1]) ;

perror=error;       //stores previous error

error = 1536 - position;

//PID

/*

Done 10000 from 30000 because iterm once saturates becuse unhanlable .

*/

// limits bound on iterm.

if (iterm>=ITERM_LIMIT)        //32767 is max limit of int

{

//To buffer the fast responses of propotional control, we have the integral control.

iterm = ITERM_LIMIT;

//LED4=~LED4;

}

else if (iterm<=-ITERM_LIMIT)        //32767 is max limit of int

{

//To buffer the fast responses of propotional control, we have the integral control.

iterm = -ITERM_LIMIT;

//LED4=~LED4;

}

else

{

iterm += error/20;

}

correction = (kp*error)  +  (ki *iterm) + (kd*(perror-error));

//anti wind up circutary

if (correction >= MAX_CORRECTION)

{

correction = MAX_CORRECTION;

}

if (correction <= MIN_CORRECTION)

{

correction = MIN_CORRECTION;

}

// 0      1         ...     5         6

// -ve correction means -ve error (position-6) ie turn right

// +ve correction means +ve error (position-0) ie turn left.

servo_output = SET_POINT-(unsigned int)correction;

servo_set (servo_output);

}

}

}

}

Assosiated header files

atd.h

void ATD_init()

{

// New code to implement interrupts

//ATD0CTL2 registers

ATD0CTL2_ADPU=1;   //ATD Power Up

ATD0CTL2_AFFC=1;   //The flag CCF clears automatically if we read the appropriate result register

//No need to use it. It will decrease the response of the system

//ATD0CTL2_AFFC=0;   //The flag clears when we read the ATDSTAT1

ATD0CTL2_ASCIE=1;  //The interrupt will be called when ASCIF is 1. Clearing this flag is necessary.

//ATD0CTL3

//ATD0CTL3_S8C=1;           // 8 adc channels sequence

ATD0CTL3_S8C=0;           // 7 ADC channels sequence

ATD0CTL3_S4C=1;           // from 0-7

ATD0CTL3_S2C=1;           //

ATD0CTL3_S1C=1;           //

//ATD0CTL4

ATD0CTL4_SRES8=1;  //8 bit resoluation

//Fastest possible settings already

ATD0CTL4_SMP=0;     //2 clock cycles.

ATD0CTL4_PRS=0;    //divinde BUS clock by 2. However the MAX BUS clock is 4mhz and min is 2mhx

//ATD0CTL5

ATD0CTL5_DJM=0;     //Left Justified

ATD0CTL5_MULT=1;

ATD0CTL5_SCAN=0;    //single conversion mode

//ATD0CTL5_SCAN=1;    //CONTINUOUS mode

}

Compensation.h

/*Compensation ratio = 256.0 / (Black value – White value)*/

float com_ratio[7]=

{

1.75342, 2.0645, 2.39252, 2.3486, 2.4151, 2.0983, 1.9845

};

// MAX VALUES FOR w_val. SEPARATE w_val COMPUTATION ALGORITHM

unsigned char w_val[7]=

{

11, 10, 10, 10, 10, 10, 10

};

// MIN VALUES FOR b_val. SEPARATE b_val COMPUTATION ALGORITHM

unsigned char b_val[7] =

{

157, 134, 117, 119, 116, 132, 139

};

void compensate_value(unsigned char* val,unsigned char* com_val)

{

int i;

for(i=0;i<6;i++)

{

if(*(val+i)<w_val[i])        //lower bound

{

*(com_val+i)=0;

}

else if (*(val+i)> (b_val[i]))    //higher bound

{

*(com_val+i)=255;

}

else

{

// This will round off the value properly ....

*(com_val+i) = (unsigned char)(((*(val+i) - w_val[i]) * com_ratio[i]) + 0.5);

}

}

}

/**********************************************

Function - sort_array

Input - One compensated array

Output - Null

***********************************************

unsigned char sort_array(unsigned char* a)

{

int i,index;

index=0;

for(i=1;i<7;i++)

{

if(*(a+index) < *(a+i))

{

index=i;

}

}

return index;

}

dcmotor.h

/*

This function sets the motor speed by the use of PWM.

The argument to this function val 0<val<=200.

*/

void dc_motor_speed(unsigned char val)

{

//PORTE_PE3=0;	//Disabling the Motor driver

PWMDTY0 = val;

PWMDTY1 = 0;

//PORTE_PE3=0;	//Re-enabling the motor driver

}

void dc_motor_init()

{

//Drives the motors in forward motion..

DDRE_DDRE2=1;              // Port E pin 2 & 3 set to output

DDRE_DDRE3=1;

DDRP_DDRP0=1;

DDRP_DDRP1=1;

PORTE_PE2=0;

PORTE_PE3=1;

PWMPOL_PPOL0=1;              //PWM pulse High at begining of Period

PWMPOL_PPOL1=1;              //PWM pulse High at begining of Period

PWMCLK_PCLK0=1;              // clock SA as clock source for PWM

PWMPRCLK =0x00;              //clock A = 2MHz  clockB = 2MHz

PWMSCLA =5;                //clock  SA = clock A / (2 * 5) = 200KHz

PWMSCLB =5;                //clock  SB = clock B / (2 * 5) = 200KHz

PWMPER0 = 200;               // PWM Period           1KHz

PWMPER1 = 200;               // PWM Period           1KHz

PWMDTY0 = 70;

PWMDTY1 = 0;

PWME_PWME0=1;                //PWM channel 0 Enable

PWME_PWME1=1;                //PWM channel 1 Enable

}

void dc_motor_stop()

{

// Stop Motor

PORTE_PE3=0;

PTP_PTP0=0;

PTP_PTP1=0;

PWME_PWME0=0;                //PWM channel disable

PWME_PWME1=0;

}

void pulse_counter_init()

{

PBCTL_PBEN=1;              //16 bit pulse accumulator enable bit (cascading two 8 bit pulse accumulator)

TCTL4_EDG0B=1;        //configure the edge detector circuits

TCTL4_EDG0A=0;        //10 is capture on falling edges only..

DLYCT_DLY0=1;         //introduces delay of 1024 bus clock cycles..

DLYCT_DLY1=1;

PACN10=0;   //Sets the data register to 0.

}

eeprom.h

//Global definations

#define ON 1;

#define OFF 0;

#define LED_ON 0

#define LED_OFF 1

#define LED1 PTT_PTT4

#define LED2 PTT_PTT5

#define LED3 PTT_PTT6

#define LED4 PTT_PTT7

#define ADT_LED  LED1

#define BUTTON_LED  LED2

#define SW2  PTP_PTP5

#define SW3  PTP_PTP7

// EEPROMs VETOR ADDRESS

#define EEPROM_KP ((IEE1_TAddress)0x13F000)

#define EEPROM_KI ((IEE1_TAddress)0x13F010)      // 2 byte above

#define EEPROM_KD ((IEE1_TAddress)0x13F020)      // 2 byte above

#define EEPROM_DELAYDC ((IEE1_TAddress)0x13F030)        //Need 1 byte address..

#define EEPROM_SETPOINT ((IEE1_TAddress)0x13F038)        //Need 1 byte address..

#define EEPROM_START ((IEE1_TAddress)0x13F040)        //Need 1 byte address..

//Global Variables

union long_to_float

{

unsigned long long_val;

float float_val;

};

unsigned char START=1;        //Variable that starts the car..

// SWITCH interrupt CONTROL EEPROM VARIABLES. ...

float kp=0.0,ki=0.0,kd=0.0;

unsigned char delay_dc=3;     //variable related to START_CROSS detection

unsigned char setpoint =1;    //set point is +ve and generally small values..   VAR RELATED TO PULSE counter

void Leds_and_Switches_Init()

{

DDRT=0xF0;                // Setting Port T pin 4,5,6,7 as output

DDRP_DDRP5=0;                 //Port P Pin 5 & 7 set to input

DDRP_DDRP7=0;

PERP_PERP5=1;                 //Port P Pin 5 & 7 Pullup Enable

PERP_PERP7=1;

LED1=LED_OFF;

LED2=LED_OFF;

LED3=LED_OFF;

LED4=LED_OFF;

}

void Delay(unsigned char a)

{

unsigned int i,j;

for(j=1;j<=a;j++)            //Delay

for(i=0;i<=60000;i++);

}

pid.h

#define __DI()  { asm sei; }      /* Disable global interrupts  */

#define __EI()  { asm cli; }      /* Enable global interrupts */

#define ISR(x) __interrupt void x(void)

// variable assignments...

unsigned char sen_data[8]=

{0, 0, 0,0, 0, 0, 0,0};

/* float akps=0.11,skis=0.005;          //0.220

*/

unsigned char flag_sm=0;

char selection = -1;

int i=0;

void ISR_init()

{

__EI()    //enable interrupts

/* IRQCR: IRQEN=0 */

IRQCR &= (unsigned char)~64;

/* PIEP: PIEP7=1,PIEP6=0,PIEP5=1,PIEP4=0,PIEP3=0,PIEP2=0,PIEP1=0,PIEP0=0 */

PIEP = 160;

//new code ends

}

ISR (isr_default)

{}

ISR (isrVatd0)

{

//Clearing ASCIF is necessary as this will not cause reinitiating

//uart0_tx_newline();

//uart0_tx_newline();

//uart0_tx_newline();

//uart0_tx_string(" :::: Interrupt ADC complete haha !! :::");

//uart0_tx_newline();

//uart0_tx_newline();

//uart0_tx_newline();

// See how many reads are req to clear the reg. It is conjectured that atleast one may req..

//ADT_LED=~ADT_LED;

//Actual code

sen_data[0] =  ATD0DR0H;                //This will clear the Appropriate FLAG .....

sen_data[1] =  ATD0DR1H;

sen_data[2] =  ATD0DR2H;

sen_data[3] =  ATD0DR3H;

sen_data[4] =  ATD0DR4H;

sen_data[5] =  ATD0DR5H;

sen_data[6] =  ATD0DR6H;

sen_data[7] =  ATD0DR7H;

}

ISR (simar)

{

if(flag_sm!=0)              //flag_SELECTION_MADE

{

//selection has been made     ((( :::: button5 is INC   and button7 is DEC :::: )))

switch(selection)

{

case 0:

{

//START

if(PIFP&(1<<5))                             // (Active Low)  on Button 2 (PIN 5) has occured

{

START=~START;

// No need to toggle the LED as feedback will be from from the motors..

}

else

{

//writes to EEPROM..

unsigned char return_val;

return_val = IEE1_SetByte(EEPROM_START,START);

if(return_val == ERR_OK)

{

LED1=LED_OFF;

Delay(2);

LED1=LED_ON;

}

}

}

case 1:

{

// KI

union long_to_float simar_read;

unsigned char return_val;

if(PIFP&(1<<5))                             // (Active Low)  on Button 2 (PIN 5) has occured (INC)

{

//uart0_tx_string(" Interrupt !! ... BUTTON 2  :::: ::::: ");

//uart0_tx_newline();

// Clears flag. Writes logical one to the 5h

//PIFP = 32;

ki=ki+0.000250;

}

else

{

//uart0_tx_string(" Interrupt !! ... BUTTON 3  :::: ::::: ");

//uart0_tx_newline();

//PIFP = 128;                            // Clears flag. Writes logical one to the 7h

ki=ki-0.000250;

}

//PIFP = 160;                            // Clears flag. Writes logical one to the 5h pin n 7th pin..

// This code writes the new value of KP to EEPROM

simar_read.float_val = ki;

return_val = IEE1_SetLong(EEPROM_KI,simar_read.long_val);

if(return_val == ERR_OK)

{

LED1=LED_OFF;

Delay(2);

LED1=LED_ON;

}

//ATD0CTL5_SCAN=0;

break;

}

case 3:

{

//KP

union long_to_float simar_read;

unsigned char return_val;

if(PIFP&(1<<5))                             // (Active Low)  on Button 2 (PIN 5) has occured

{

//uart0_tx_string(" Interrupt !! ... BUTTON 2  :::: ::::: ");

//uart0_tx_newline();

// Clears flag. Writes logical one to the 5h

//PIFP = 32;

kp=kp+0.0025;

}

else

{

//uart0_tx_string(" Interrupt !! ... BUTTON 3  :::: ::::: ");

//uart0_tx_newline();

//PIFP = 128;                            // Clears flag. Writes logical one to the 7h

kp=kp-0.0025;

}

//PIFP = 160;                            // Clears flag. Writes logical one to the 5h pin n 7th pin..

// This code writes the new value of KP to EEPROM

simar_read.float_val = kp;

return_val = IEE1_SetLong(EEPROM_KP,simar_read.long_val);

if(return_val == ERR_OK)

{

LED2=LED_OFF;

Delay(2);

LED2=LED_ON;

}

break;

}

case 4:

{

// QNAN EEPROM RESET

if(PIFP&(1<<7))     //Button 3          //decrement

{

EEPROM_reset();

}

break;

}

case 5:                 //LED 3

{

// DELAY_DC

unsigned char return_val;

if(PIFP&(1<<5))                             // (Active Low)  on Button 2 (PIN 5) has occured

{

//uart0_tx_string(" Interrupt !! ... BUTTON 2  :::: ::::: ");

//uart0_tx_newline();

// Clears flag. Writes logical one to the 5h

//PIFP = 32;

delay_dc++;

}

else

{

//uart0_tx_string(" Interrupt !! ... BUTTON 3  :::: ::::: ");

//uart0_tx_newline();

//PIFP = 128;                            // Clears flag. Writes logical one to the 7h

delay_dc--;

}

//PIFP = 160;                            // Clears flag. Writes logical one to the 5h pin n 7th pin..

// This code writes the new value of KP to EEPROM

return_val = IEE1_SetByte(EEPROM_DELAYDC,delay_dc);

if(return_val == ERR_OK)

{

LED3=LED_OFF;

Delay(2);

LED3=LED_ON;

}

break;

}

case 6:                //LED2

{

// SETPOINT

unsigned char return_val;

if(PIFP&(1<<5))                             // (Active Low)  on Button 2 (PIN 5) has occured

{

//uart0_tx_string(" Interrupt !! ... BUTTON 2  :::: ::::: ");

//uart0_tx_newline();

// Clears flag. Writes logical one to the 5h

//PIFP = 32;

setpoint++;

}

else

{

//uart0_tx_string(" Interrupt !! ... BUTTON 3  :::: ::::: ");

//uart0_tx_newline();

//PIFP = 128;                            // Clears flag. Writes logical one to the 7h

setpoint--;

}

//PIFP = 160;                            // Clears flag. Writes logical one to the 5h pin n 7th pin..

// This code writes the new value of KP to EEPROM

return_val = IEE1_SetByte(EEPROM_SETPOINT,setpoint);

if(return_val == ERR_OK)

{

LED2=LED_OFF;

Delay(2);

LED2=LED_ON;

}

break;

}

case 7:             //black compensattion

{

START=0;            //STOPs the car.

if(PIFP&(1<<5))                             // (Active Low)  on Button 2 (PIN 5) has occured

{

//Start a new conversion

for(i=0;i<=6;i++)

{

if(b_val[i]<sen_data[i])

{

//write new max value to b_val[i]

b_val[i]=sen_data[i];

}

}

ATD0CTL5_SCAN=0;          //This will start a new conversion ....

}

else

{

//make it final

//write the data to eeprom

//test it on computer before writing this code...

}

break;

}

case 8:             //white compensattion

{

START=0;            //STOPs the car.

if(PIFP&(1<<5))                             // (Active Low)  on Button 2 (PIN 5) has occured

{

//Start a new conversion

for(i=0;i<=6;i++)

{

if(w_val[i]<sen_data[i])

{

//write new max value to b_val[i]

w_val[i]=sen_data[i];

}

}

ATD0CTL5_SCAN=0;          //This will start a new conversion ....

}

else

{

//make it final

//write the data to eeprom

//test it on computer before writing this code...

}

break;

}

}

}

else

{

if(PIFP&(1<<5))                             // MENU SCROLLER

{

if(selection>=5)    //Rotating menu..

{

selection=0;

}

else

{

selection++;

}

switch(selection)

{

case 0:                                         //LED1      //start

{

LED1=LED_ON;

LED2=LED_OFF;

LED3=LED_OFF;

LED4=LED_OFF;

break;

}

case 1:                              //LED2                            ki

{

LED1=LED_OFF;

LED2=LED_ON;

LED3=LED_OFF;

LED4=LED_OFF;

break;

}

case 2:   //                                      LED3                    kp

{

LED1=LED_OFF;

LED2=LED_OFF;

LED3=LED_ON;

LED4=LED_OFF;

break;

}

case 3:             //kd

{

LED1=LED_OFF;

LED2=LED_OFF;

LED3=LED_OFF;

LED4=LED_ON;

break;

}

case 4:        //QNAN                            LED4       ToDo change in reset

{

//DONT CHANGE ITS INDEX NO ... EEMPROM_RESET HAS TO BE CHANGES

LED1=LED_OFF;

LED2=LED_OFF;

LED3=LED_ON;

LED4=LED_OFF;

break;

}

case 5:   //bounce back mechanism       LED2      delay_dc

{

LED1=LED_OFF;

LED2=LED_ON;

LED3=LED_OFF;

LED4=LED_OFF;

break;

}

case 6:   //bounce back mechanism       LED1                    setpoint

{

LED1=LED_ON;

LED2=LED_OFF;

LED3=LED_OFF;

LED4=LED_OFF;

}

case 7:   //bounce back mechanism       LED2                  black   compensation

{

LED1=LED_OFF;

LED2=LED_ON;

LED3=LED_OFF;

LED4=LED_OFF;

//start ADT conversion

ATD0CTL5_SCAN=0;          //This will start a new conversion ....

break;

//TAKE THE PRECAUTION OF KEEPING THE CAR ON THE TRACK RIGHT BEFORE

//SELECTING THE MENU ....

}

case 8:   //bounce back mechanism       LED3                   white  compensation

{

LED1=LED_OFF;

LED2=LED_OFF;

LED3=LED_ON;

LED4=LED_OFF;

//start ADT conversion

ATD0CTL5_SCAN=0;          //This will start a new conversion ....

break;

//TAKE THE PRECAUTION OF KEEPING THE CAR ON THE TRACK RIGHT BEFORE

//SELECTING THE MENU ....

}

}

}

else

{

//7th button is menu SELECTOR

flag_sm=1;

}

}

Delay(1);

PIFP = 160;                            // Clears flag. Writes logical one to the 5h pin n 7th pin..

Delay(1);

}

/* Interrupt vector table */

/* ISR prototype */

typedef void (*near tIsrFunc)(void);

#ifndef UNASSIGNED_ISR

#define UNASSIGNED_ISR isr_default   /* unassigned interrupt service routine */

#endif

const tIsrFunc _InterruptVectorTable[] @0xFF10 = { /* Interrupt vector table */

/* ISR name                               No.  Address Pri XGATE Name          Description */

UNASSIGNED_ISR,                       /* 0x08  0xFF10   -   -    ivVsi         unused by PE */

UNASSIGNED_ISR,                       /* 0x09  0xFF12   1   no   ivReserved119 unused by PE */

UNASSIGNED_ISR,                       /* 0x0A  0xFF14   1   no   ivReserved118 unused by PE */

UNASSIGNED_ISR,                       /* 0x0B  0xFF16   1   no   ivReserved117 unused by PE */

UNASSIGNED_ISR,                       /* 0x0C  0xFF18   1   no   ivReserved116 unused by PE */

UNASSIGNED_ISR,                       /* 0x0D  0xFF1A   1   no   ivReserved115 unused by PE */

UNASSIGNED_ISR,                       /* 0x0E  0xFF1C   1   no   ivReserved114 unused by PE */

UNASSIGNED_ISR,                       /* 0x0F  0xFF1E   1   no   ivReserved113 unused by PE */

UNASSIGNED_ISR,                       /* 0x10  0xFF20   1   no   ivReserved112 unused by PE */

UNASSIGNED_ISR,                       /* 0x11  0xFF22   1   no   ivReserved111 unused by PE */

UNASSIGNED_ISR,                       /* 0x12  0xFF24   1   no   ivReserved110 unused by PE */

UNASSIGNED_ISR,                       /* 0x13  0xFF26   1   no   ivReserved109 unused by PE */

UNASSIGNED_ISR,                       /* 0x14  0xFF28   1   no   ivReserved108 unused by PE */

UNASSIGNED_ISR,                       /* 0x15  0xFF2A   1   no   ivReserved107 unused by PE */

UNASSIGNED_ISR,                       /* 0x16  0xFF2C   1   no   ivReserved106 unused by PE */

UNASSIGNED_ISR,                       /* 0x17  0xFF2E   1   no   ivReserved105 unused by PE */

UNASSIGNED_ISR,                       /* 0x18  0xFF30   1   no   ivReserved104 unused by PE */

UNASSIGNED_ISR,                       /* 0x19  0xFF32   1   no   ivReserved103 unused by PE */

UNASSIGNED_ISR,                       /* 0x1A  0xFF34   1   no   ivReserved102 unused by PE */

UNASSIGNED_ISR,                       /* 0x1B  0xFF36   1   no   ivReserved101 unused by PE */

UNASSIGNED_ISR,                       /* 0x1C  0xFF38   1   no   ivReserved100 unused by PE */

UNASSIGNED_ISR,                       /* 0x1D  0xFF3A   1   no   ivReserved99  unused by PE */

UNASSIGNED_ISR,                       /* 0x1E  0xFF3C   1   no   ivReserved98  unused by PE */

UNASSIGNED_ISR,                       /* 0x1F  0xFF3E   1   no   ivReserved97  unused by PE */

UNASSIGNED_ISR,                       /* 0x20  0xFF40   1   no   ivReserved96  unused by PE */

UNASSIGNED_ISR,                       /* 0x21  0xFF42   1   no   ivReserved95  unused by PE */

UNASSIGNED_ISR,                       /* 0x22  0xFF44   1   no   ivReserved94  unused by PE */

UNASSIGNED_ISR,                       /* 0x23  0xFF46   1   no   ivReserved93  unused by PE */

UNASSIGNED_ISR,                       /* 0x24  0xFF48   1   no   ivReserved92  unused by PE */

UNASSIGNED_ISR,                       /* 0x25  0xFF4A   1   no   ivReserved91  unused by PE */

UNASSIGNED_ISR,                       /* 0x26  0xFF4C   1   no   ivReserved90  unused by PE */

UNASSIGNED_ISR,                       /* 0x27  0xFF4E   1   no   ivReserved89  unused by PE */

UNASSIGNED_ISR,                       /* 0x28  0xFF50   1   no   ivReserved88  unused by PE */

UNASSIGNED_ISR,                       /* 0x29  0xFF52   1   no   ivReserved87  unused by PE */

UNASSIGNED_ISR,                       /* 0x2A  0xFF54   1   no   ivReserved86  unused by PE */

UNASSIGNED_ISR,                       /* 0x2B  0xFF56   1   no   ivReserved85  unused by PE */

UNASSIGNED_ISR,                       /* 0x2C  0xFF58   1   no   ivReserved84  unused by PE */

UNASSIGNED_ISR,                       /* 0x2D  0xFF5A   1   no   ivReserved83  unused by PE */

UNASSIGNED_ISR,                       /* 0x2E  0xFF5C   1   no   ivReserved82  unused by PE */

UNASSIGNED_ISR,                       /* 0x2F  0xFF5E   1   no   ivReserved81  unused by PE */

UNASSIGNED_ISR,                       /* 0x30  0xFF60   1   -    ivVxsramav    unused by PE */

UNASSIGNED_ISR,                       /* 0x31  0xFF62   1   -    ivVxsei       unused by PE */

UNASSIGNED_ISR,                       /* 0x32  0xFF64   1   no   ivVxst7       unused by PE */

UNASSIGNED_ISR,                       /* 0x33  0xFF66   1   no   ivVxst6       unused by PE */

UNASSIGNED_ISR,                       /* 0x34  0xFF68   1   no   ivVxst5       unused by PE */

UNASSIGNED_ISR,                       /* 0x35  0xFF6A   1   no   ivVxst4       unused by PE */

UNASSIGNED_ISR,                       /* 0x36  0xFF6C   1   no   ivVxst3       unused by PE */

UNASSIGNED_ISR,                       /* 0x37  0xFF6E   1   no   ivVxst2       unused by PE */

UNASSIGNED_ISR,                       /* 0x38  0xFF70   1   no   ivVxst1       unused by PE */

UNASSIGNED_ISR,                       /* 0x39  0xFF72   1   no   ivVxst0       unused by PE */

UNASSIGNED_ISR,                       /* 0x3A  0xFF74   1   no   ivVpit3       unused by PE */

UNASSIGNED_ISR,                       /* 0x3B  0xFF76   1   no   ivVpit2       unused by PE */

UNASSIGNED_ISR,                       /* 0x3C  0xFF78   1   no   ivVpit1       unused by PE */

UNASSIGNED_ISR,                       /* 0x3D  0xFF7A   1   no   ivVpit0       unused by PE */

UNASSIGNED_ISR,                       /* 0x3E  0xFF7C   1   no   ivVReserved65 unused by PE */

UNASSIGNED_ISR,                       /* 0x3F  0xFF7E   1   no   ivVapi        unused by PE */

UNASSIGNED_ISR,                       /* 0x40  0xFF80   1   no   ivVlvi        unused by PE */

UNASSIGNED_ISR,                       /* 0x41  0xFF82   1   no   ivVReserved62 unused by PE */

UNASSIGNED_ISR,                       /* 0x42  0xFF84   1   no   ivVReserved61 unused by PE */

UNASSIGNED_ISR,                       /* 0x43  0xFF86   1   no   ivVsci4       unused by PE */

UNASSIGNED_ISR,                       /* 0x44  0xFF88   1   no   ivVReserved59 unused by PE */

UNASSIGNED_ISR,                       /* 0x45  0xFF8A   1   no   ivVsci2       unused by PE */

UNASSIGNED_ISR,                       /* 0x46  0xFF8C   1   no   ivVpwmesdn    unused by PE */

simar,                                /* 0x47  0xFF8E   1   no   ivVportp      used by PE */

UNASSIGNED_ISR,                       /* 0x48  0xFF90   1   no   ivVReserved55 unused by PE */

UNASSIGNED_ISR,                       /* 0x49  0xFF92   1   no   ivVReserved54 unused by PE */

UNASSIGNED_ISR,                       /* 0x4A  0xFF94   1   no   ivVReserved53 unused by PE */

UNASSIGNED_ISR,                       /* 0x4B  0xFF96   1   no   ivVReserved52 unused by PE */

UNASSIGNED_ISR,                       /* 0x4C  0xFF98   1   no   ivVReserved51 unused by PE */

UNASSIGNED_ISR,                       /* 0x4D  0xFF9A   1   no   ivVReserved50 unused by PE */

UNASSIGNED_ISR,                       /* 0x4E  0xFF9C   1   no   ivVReserved49 unused by PE */

UNASSIGNED_ISR,                       /* 0x4F  0xFF9E   1   no   ivVReserved48 unused by PE */

UNASSIGNED_ISR,                       /* 0x50  0xFFA0   1   no   ivVReserved47 unused by PE */

UNASSIGNED_ISR,                       /* 0x51  0xFFA2   1   no   ivVReserved46 unused by PE */

UNASSIGNED_ISR,                       /* 0x52  0xFFA4   1   no   ivVReserved45 unused by PE */

UNASSIGNED_ISR,                       /* 0x53  0xFFA6   1   no   ivVReserved44 unused by PE */

UNASSIGNED_ISR,                       /* 0x54  0xFFA8   1   no   ivVReserved43 unused by PE */

UNASSIGNED_ISR,                       /* 0x55  0xFFAA   1   no   ivVReserved42 unused by PE */

UNASSIGNED_ISR,                       /* 0x56  0xFFAC   1   no   ivVReserved41 unused by PE */

UNASSIGNED_ISR,                       /* 0x57  0xFFAE   1   no   ivVReserved40 unused by PE */

UNASSIGNED_ISR,                       /* 0x58  0xFFB0   1   no   ivVcan0tx     unused by PE */

UNASSIGNED_ISR,                       /* 0x59  0xFFB2   1   no   ivVcan0rx     unused by PE */

UNASSIGNED_ISR,                       /* 0x5A  0xFFB4   1   no   ivVcan0err    unused by PE */

UNASSIGNED_ISR,                       /* 0x5B  0xFFB6   1   no   ivVcan0wkup   unused by PE */

UNASSIGNED_ISR,                       /* 0x5C  0xFFB8   1   no   ivVflash      unused by PE */

UNASSIGNED_ISR,                       /* 0x5D  0xFFBA   1   no   ivVeeprom     unused by PE */

UNASSIGNED_ISR,                       /* 0x5E  0xFFBC   1   no   ivVspi2       unused by PE */

UNASSIGNED_ISR,                       /* 0x5F  0xFFBE   1   no   ivVspi1       unused by PE */

UNASSIGNED_ISR,                       /* 0x60  0xFFC0   1   no   ivViic0       unused by PE */

UNASSIGNED_ISR,                       /* 0x61  0xFFC2   1   no   ivVReserved30 unused by PE */

UNASSIGNED_ISR,                       /* 0x62  0xFFC4   1   no   ivVcrgscm     unused by PE */

UNASSIGNED_ISR,                       /* 0x63  0xFFC6   1   no   ivVcrgplllck  unused by PE */

UNASSIGNED_ISR,                       /* 0x64  0xFFC8   1   no   ivVtimpabovf  unused by PE */

UNASSIGNED_ISR,                       /* 0x65  0xFFCA   1   no   ivVtimmdcu    unused by PE */

UNASSIGNED_ISR,                       /* 0x66  0xFFCC   1   no   ivVporth      unused by PE */

UNASSIGNED_ISR,                       /* 0x67  0xFFCE   1   no   ivVportj      unused by PE */

UNASSIGNED_ISR,                       /* 0x68  0xFFD0   1   no   ivVatd1       unused by PE */

isrVatd0,                             /* 0x69  0xFFD2   1   no   ivVatd0       used by PE */

UNASSIGNED_ISR,                       /* 0x6A  0xFFD4   1   no   ivVsci1       unused by PE */

UNASSIGNED_ISR,                       /* 0x6B  0xFFD6   1   no   ivVsci0       unused by PE */

UNASSIGNED_ISR,                       /* 0x6C  0xFFD8   1   no   ivVspi0       unused by PE */

UNASSIGNED_ISR,                       /* 0x6D  0xFFDA   1   no   ivVtimpaie    unused by PE */

UNASSIGNED_ISR,                       /* 0x6E  0xFFDC   1   no   ivVtimpaaovf  unused by PE */

UNASSIGNED_ISR,                       /* 0x6F  0xFFDE   1   no   ivVtimovf     unused by PE */

UNASSIGNED_ISR,                       /* 0x70  0xFFE0   1   no   ivVtimch7     unused by PE */

UNASSIGNED_ISR,                       /* 0x71  0xFFE2   1   no   ivVtimch6     unused by PE */

UNASSIGNED_ISR,                       /* 0x72  0xFFE4   1   no   ivVtimch5     unused by PE */

UNASSIGNED_ISR,                       /* 0x73  0xFFE6   1   no   ivVtimch4     unused by PE */

UNASSIGNED_ISR,                       /* 0x74  0xFFE8   1   no   ivVtimch3     unused by PE */

UNASSIGNED_ISR,                       /* 0x75  0xFFEA   1   no   ivVtimch2     unused by PE */

UNASSIGNED_ISR,                       /* 0x76  0xFFEC   1   no   ivVtimch1     unused by PE */

UNASSIGNED_ISR,                       /* 0x77  0xFFEE   1   no   ivVtimch0     unused by PE */

UNASSIGNED_ISR,                       /* 0x78  0xFFF0   1   no   ivVrti        unused by PE */

UNASSIGNED_ISR,                       /* 0x79  0xFFF2   1   no   ivVirq        unused by PE */

UNASSIGNED_ISR,                       /* 0x7A  0xFFF4   -   -    ivVxirq       unused by PE */

UNASSIGNED_ISR,                       /* 0x7B  0xFFF6   -   -    ivVswi        unused by PE */

UNASSIGNED_ISR                        /* 0x7C  0xFFF8   -   -    ivVtrap       unused by PE */

};

servomotor.h

void servo_centre()

{

//	Values of registers is 1415

PWMDTY2 = 0x05;                // Pulse Width 1.5ms: Center Position

//	 PWMDTY3 = 0x87;                //1.415ms

// Value of register os 1500....

PWMDTY3 = 0xDC;                //1.5ms

}

void servo_init()

{

PWMCTL_CON23=1;

PWME_PWME2=1;                //PWM channel 2 Enable

PWME_PWME3=1;                //PWM channel 3 Enable

PWMPOL_PPOL2=1;              //PWM pulse High at begining of Period

PWMPOL_PPOL3=1;              //PWM pulse High at begining of Period

PWMCLK_PCLK2=1;              // clock SA as clock source for PWM

PWMCLK_PCLK3=1;              // clock SB as clock source for PWM

PWMPRCLK =0x00;              //clock A = 2MHz  clockB = 2MHz

// Time of this clock pulse is 1 us

PWMSCLA =1;                //clock  SA = clock A / (2 * 1) = 1MHz

PWMSCLB =1;                //clock  SB = clock B / (2 * 1) = 1MHz

// Value of the register as PWMPER2-PWMPER3 is 20,000.

// So the value of period of the flipping is 20ms

PWMPER2 = 0x4E;               // PWM Period 20ms           50Hz

PWMPER3 = 0x20;               // PWM Period 20ms           50Hz

servo_centre();

}

void servo_left()

{

//	Values of register is 1100

PWMDTY2 = 0x04;                //Pulse Width: Left Position

PWMDTY3 = 0x6A;                //1.1ms

}

void servo_right()

{

//	Values of register is 1700

PWMDTY2 = 0x06;                //Pulse Width 1.7 ms: Right Position

PWMDTY3 = 0xA4;

}

void servo_set(unsigned int a)

{

//	a should range somewhere in 1100 to 1700

PWMDTY2 = (a>>8);

a=a<<8;

a=a>>8;

PWMDTY3 = a;

}

Development tool and debugging process

As debugging in embedded system is a very important part, we tried to put our maximum effort here.

We created software like Data Logger when we felt like exporting sensor values to excel to study them better using mathematical tool there. We plotted graphs using the sensor position data and identified certain very significant loopholes in the current position finding algorithm. The quantized nature of the default position finding algorithm drove us towards introducing some other better version but unfortunately, time did not allowed us to do so. However the software did helped us a lot in understanding the sensor values, finding mix, max and in the end proved to be a boon to us.

The main problem with the Data Logger was that, we were not able to see the data in real time and also the software is not so portable. As need is the force to creation and innovation of ideas, further exploration towards the end, we encountered code warrior virtualization tool. So we had another powerful tool and it helped us to better debug the software.

- Codewarrior

- Freescale’s student  development module

- Data Logger – made using visual basic

- Codewarriors’s virtual tool

While the codewarrior was provided to us by Freescale itself. The data logger software was made exclusively for logging the sensor data coming from the ATD converter into a spreadsheet from where the desired functions on the data can be implemented.

Starting with simple algorithm to run the robot on straight path, the code became more and more complex  to run the robot effectively on an oval track. Further improvements involve increasing the speed of the robot and implementing a better control system to minimize the oscillations and maximize the speed .

SCREENSHOTS:

Observing the values from the serial port and deriving conclusions

Data Logger software interface

Date logged into a spreadsheet by data logger

Drawing conclusions from the data logged

Virtualization Tool

Codewarrior coding section

Codewarrior debugging tool

Stages of  debugging

The code was written and compiled in C using the codewarriors in 10 different stages:

STAGE 1: getting familiar with the servomotor

Stage 1 started with taking control over the servo motor. The servo motor was given PWM duty cycle using serial cable and by incrementing and decrementing the PWM duty cycle to get the value for rightmost and leftmost turn. The value was actually a 16 bit value stored in two registers by right and left shifting using the code:

void servo_set(unsigned int a)

{

//	a should range somewhere in 1100 to 1700

PWMDTY2 = (a>>8);

a=a<<8;

a=a>>8;

PWMDTY3 = a;

}

The leftmost value truned out to be : 1205

Rightmost value =1815

Central value = 1510

STAGE 2: compensating the sensor reading

Stage 2 included taking sensor values from the ATD converter through serial cable connection by the help of data logger. The sensor values were collected and calibrated.

STAGE 3: implementing PI controller

Stage 3 comprises of implementing PI control and running the robot with PI controller. The error values were checked in this stage to be right and according to the used algorithm.

STAGE 4: completing oval track

Stage 4  include first completely working code running the robot on an oval track. in this stage the robot ran around an oval track for the first time.

STAGE 5: implementing writing to  EEPROM

In stage 5 writing on  EEPROM was implementing so that the tuning can be done in real time and all the P and I values were strored in the EEPROM and they could be changed by using the buttons provided and auto compensation of  sensor values was done.

STAGE 6: upgrading to PID controller

In this stage PI control was upgraded to PID controller.

STAGE 7:  Programming to stop after 2 laps

in this stage the programme was provided with a code section to stop the car after 2 laps.

STAGE 8:  Menu Interface

Stage 8 included making a menu interface to effectively tune the car and store the result in EEPROM. The menu works with the help of two buttons .The button 2 was used to scroll through the menu items and a corresponding LED pattern is displayed. Once the desired option is obtained the button 3 is pressed to select it and then button 3 is pressed for further requirements. But while increasing the Kp, Ki or Kd values button 3  decrements and button 2 increments these values.

Menu interface:

| Menu Scrolling by Button 2 | Button 3 | Button3 | Button2 |
| --- | --- | --- | --- |
| compensation | selects | Black compensation | White compensation |
| Ki | Selects | Decrement | Increment |
| Kd | Selects | Decrement | Increment |
| Kp | Selects | Decrement | increment |
| EEPROM resets | selects | confirms | Do nothing |
| Delay dc (for tuning the stopping point) | selects | Decrement | increment |

STAGE 9: Using pulse encoder

We were facing a big problem in the previous stages that whenever we tune the robot at one speed perfectly then after sometime the battery power will go down and the spedd at which we were running would vary and hence the tuning has to be all over again. So to overcome this problem we used the pulse encoder and implemented PD controller in it to derive constant power from the battery.

STAGE 10: Floating point emulation

We tested using Real time simulation in code warrior to calculate our sampling time. What was found was a very interesting point. Using float in the program decrease the speed of processing of calculation by 100 times for one float.

This is a very significant issue so we decided to implement fixed point algorithm. Further exploring the net we discovered that the use of complex fixed point algorithm can be avoided in this processor by dividing the result by a particular number as here.

As MC9S12 series are equipped with hardware division circuitry, dividing doesn’t consume any unnecessary clock cycles and operates quite good in speed. This way we were able to increase our sampling time from 1.6ms to 0.5ms.

Sampling Time

Overall Sampling time: 0.5ms

ATD Interrupt  time : Time spent in the ATD interrupt. Since this interrupt is very frequent, the time of execution of this interrupt has to as small as possible. We made it the smallest possible and the time is 0.084ms.